CASE STUDY

Unified IP Intercom, Paging & PBX Deployment

Indian Institute of Technology Madras

| Client | Indian Institute of Technology Madras |
| --- | --- |
| Location | Chennai, Tamil Nadu 600036 |
| Engagement Type | Remote Configuration & Deployment |
| Timeline | 2 days (01-09-2026 to 02-09-2026) |
| Status | Completed — Order Billed |

Partner Chain

This deployment was delivered as a direct sale by Micro Village Communications Pvt. Ltd. (MVC), the national distributor for Grandstream Networks in India, with no intermediary system integrator involved.

| Role | Entity |
| --- | --- |
| OEM | Grandstream Networks |
| Distributor (Direct Sale) | Micro Village Communications Pvt. Ltd. (MVC) |
| End Client | Indian Institute of Technology Madras |

Client Background

The Indian Institute of Technology Madras is one of India's premier public technical institutes, with a large, multi-building campus that depends on dependable internal voice communication across academic, administrative, and residential blocks.

The Challenge

IIT Madras needed a way to connect a control-room SIP microphone console to multiple audio intercom points and a talk-back horn speaker, with one-touch calling so non-technical staff could reach any point instantly rather than dialling extensions manually. The solution had to run on a single, centrally administered IP PBX and be quick to roll out without an on-site engineer visit.

Scope of Work & Solution Deployed

MVC's engineering team remotely supplied, configured, and commissioned a UCM6300A IP PBX with SIP extensions for the M20 microphone console, both GDS3702 audio intercoms, and the GSC3518HS horn speaker. One-touch speed-dial routing was configured on the M20 mic so that:

- Press 1 → GSC3518HS speaker

- Press 2 → GDS3702 intercom (unit 1)

- Press 3 → GDS3702 intercom (unit 2)

- Press 4 → GSC3518HS speaker and GDS3702 intercom (combined paging group)

GDS3702 single-press calling and dedicated paging extensions were also configured on the UCM6300A, giving site staff group-paging capability in addition to individual point-to-point calling.

Products Deployed

| S.No | Equipment | Model | Qty |
| --- | --- | --- | --- |
| 1 | IP PBX | UCM6300A | 1 |
| 2 | Audio IP Intercom System | GDS3702 | 2 |
| 3 | Goose Neck Microphone | M20 | 1 |
| 4 | SIP/Multicast Talk-Back Horn Speaker | GSC3518HS | 1 |

| 4 Grandstream units commissioned | 2 Days Configuration to completion | 100% Remote delivery | Direct MVC-to-client sale |
| --- | --- | --- | --- |

Purpose of Deployment

- IIT Madras has a large, sprawling area of gardens and forest. To facilitate announcements regarding wildlife movement and enable people in the area to request assistance from the control room, IIT Madras approached MVC for a two-way communication solution.

- MVC designed and implemented a solution comprising a UCM, horn speakers, and single-button audio intercoms to meet the client’s requirements. This is currently a pilot deployment. Based on satisfactory performance, MVC expects to receive a larger order for similar deployments in the near future.

Testing & Verification

- Internal call testing completed across the microphone, speaker, and audio intercom extensions.

- Speed dial mapping on the M20 mic (keys 1–4) verified and confirmed working as expected.

- All UCM6300A, M20 mic, GDS3702 (×2), and GSC3518HS units confirmed operational after configuration.

Training Provided

MVC's engineer, Pankesh G., delivered hands-on training to IIT Madras site personnel covering system configuration, extension management, speed-dial configuration, and paging extension creation, so the institute's own team can manage day-to-day changes independently.

Outcome

The engagement was completed to the customer's satisfaction within two days of remote configuration, with all units operational and site personnel trained on ongoing administration. The order was billed on completion, with zero on-site engineer visits required.

Future Scope & Upcoming Opportunities

Following the successful rollout at the Chennai campus, MVC is in discussion with IIT Madras for similar Grandstream intercom, paging, and PBX deployments at other institute branches and campuses. These orders are yet to be confirmed, but the completed deployment positions MVC and Grandstream as the proven, reference-checked solution for any future expansion across the institute's other locations.